Sensor Unit L

Product Sheet v2.2

## Technical Specifications

| Feature | Details |
| --- | --- |
| Sampling Period | 1, 5, 10, 15, 30, 60 minutes |
| Unit Life | +10 years |
| Connectivity | Cellular 4G/NB-IoT w/ 2G compatibility |
| Operating Conditions | -10°C to +65°C, 10% to 99% RH |
| Dimensions | 35cm x 25cm x 18cm |
| Weight | 3.5kg with solar panel |
| Enclosure | IP34* rated |
| Powering Options | Solar panel or 5V DC power source |
| Battery | 3.7V 15.2Ah |
| Compliance | CE, Engineered to CEN/TS 17660-1 Class 1 & CEN/TS 17660-2 Performance Standards |

## Packaging Options

| Package | Included Components |
| --- | --- |
| Lite | PM1, PM2.5, PM10, Ambient Temperature, Relative Humidity and Pressure |
| Gas-1 | Lite + 1 regular Gas |
| Gas-2 | Lite + 2 regular Gas |
| Gas-3 | Lite + 3 regular Gas |
| Gas-4 | Lite + 4 regular Gas |
| Gas-Special | Contact us for custom configurations |

Customization Options

Regular Gas Sensors: NO, NO2, O3, SO2, CO
Special Gas Sensors: CO2, H2S, VOC, CH4, N2O
External Add-ons: Wind speed/direction, noise monitoring (Class-1 or Class-2), extended solar power package

*Designed with an open sampling architecture for air quality measurement; the enclosure carries an IP34 protection rating.

Sensor Performance

| Parameter | Sensor Type | Accuracy | Range |
| --- | --- | --- | --- |
| PM2.5 | Laser scattering | r² > 0.9* | 0-1000 μg/m³ (1μg/m³ resolution) |
| PM10 | Laser scattering | r² > 0.9* | 0-1000 μg/m³ (1μg/m³ resolution) |
| NO2 | Electrochemical | r² > 0.82*, LOD: 2 ppb | 0-5000 ppb (1 ppb resolution) |
| O3 | Electrochemical | r² > 0.82*, LOD: 5 ppb | 0-5000 ppb (1 ppb resolution) |
| SO2 | Electrochemical | r² > 0.7*, LOD: 5 ppb | 0-5000 ppb (1 ppb resolution) |
| CO | Electrochemical | r² > 0.8*, LOD: 10 ppb | 0-10 ppm (1 ppb resolution) |

*r²-scores calculated during co-location trials with FEM reference instruments

## Key Applications

Municipal Air Quality Networks Identify pollution hotspots and verify mitigation efforts
Industrial Compliance Monitoring Ensure regulatory compliance with accurate, continuous data
Environmental Impact Assessment Measure and validate improvements from green technology investments

Why Airqoon?

Our intelligent calibration process and cloud-based data management system transform raw measurements into actionable environmental intelligence, delivering field-proven performance against reference-grade instruments. Continuous accuracy is maintained through on-site collocation and cloud-based calibration. Designed for rapid deployment, the unit is operational within minutes, no installation expertise required.

7 W Solar Panel

Address: Marmara University Teknopark, Maltepe,

Istanbul – Turkiye

info@airqoon.com

airqoon.com

Core box

IP34

protection

Sensor box

“*”depending on the preferred sampling period and environmental conditions